Improving the time resolution of Thomson scattering via machine learning on reflectometry data

Diogo R. Ferreira1, Andreas Gillgren2, Andrei Ludvig-Osipov2, Pär Strand2, and JET Contributors*

1 Instituto de Plasmas e Fusão Nuclear, Instituto Superior Técnico, Universidade de Lisboa, 1049-001 Lisboa, Portugal

2 Department of Space, Earth and Environment, Chalmers University of Technology, SE-412 96 Gothenburg, Sweden

* See the author list of J. Mailloux et al 2022 Nucl. Fusion 62 042026

When measuring the electron density profile at JET, the preferred diagnostic is usually HRTS (high resolution Thomson scattering) [1], which provides good accuracy in terms of radial position (on the order of 1 cm) but has a low sampling rate (20 Hz) [2]. This makes it infeasible to analyze pedestal dynamics in detail, especially for phenomena such as ELMs (edge localized modes), which occur on faster time scales, and would require at least a sampling rate in the range of 100 Hz to 1 kHz to capture the transient processes associated with ELM crashes. On the other hand, the reflectometry system at JET [3] provides a high temporal resolution (on the order of 1-10 kHz) but is not as accurate in terms of the radial position of the density measurements, which led to the development of new reconstruction methods [4]. In this work, we develop a virtual diagnostic which combines the spatial accuracy of HRTS with the temporal resolution of the reflectometer diagnostic. For this purpose, we train a neural network to predict HRTS-like profiles from reflectometry data. Once trained, we observe that the model agrees closely with HRTS, and is able to overcome some problems that occur in reflectometry-based reconstructions. In addition, the model estimates the density values at the same positions as HRTS, but with a much higher time resolution, which enables the analysis and visualization of ELM dynamics throughout a pulse. We illustrate the results on the pulse that achieved the current energy record in the recent D-T (deuterium-tritium) campaign at JET.

References

[1] R. Pasqualotto et al 2004 Rev. Sci. Instrum. 75 3891

[2] L. Frassinetti et al 2012 Rev. Sci. Instrum. 83 013506

[3] A. Sirinelli et al 2010 Rev. Sci. Instrum. 81 10D939

[4] R. B. Morales et al 2017 Rev. Sci. Instrum. 88 043503

This work has been carried out within the framework of the EUROfusion Consortium, funded by the European Union via the Euratom Research and Training Programme (Grant Agreement No 101052200 — EUROfusion). Views and opinions expressed are however those of the author(s) only and do not necessarily reflect those of the European Union or the European Commission. Neither the European Union nor the European Commission can be held responsible for them.