Proposal for measurement of p-11B reaction in the EHL-2 spherical torus

Zhi LI1,2, Bing LIU1,2, Tiantian SUN1,2, Xinchen JIANG1,2, Di LUO1,2, Yingying LI1,2, Yuejiang SHI1,2, Huasheng XIE1,2, Shaodong SONG1,2, Yuankai PENG1,2

1 Hebei Key Laboratory of Compact Fusion, Langfang 065001, China

2 ENN Science and Technology Development Co., Ltd., Langfang 065001, China

EHL-2 is the next generation of spherical torus under design in the ENN Energy Research Institute towards proton-boron fusion energy. One of the important missions of EHL-2 is to achieve proton-boron (p-11B) reaction in a magnetized plasma possibly driven by external heating systems. A big challenge is the measurement of the reaction rate of these low reactivity events in the magnetic fusion environment. Here, we propose to a strategy for measuring p-11B reaction rate in EHL-2 through integrated modeling. Preliminary analysis shows that considering the H-B plasma in EHL-2, with  electron density of ne=1.3×1020 m−3, ion temperature of Ti=30 keV and nB=nH/9, more than 1015 alpha particles could be produced. Such a number of alpha particles is able to be detected and measured by varies types of detectors (i.e. FILD, NPA, GRS, class of CR39, etc.). A further simulation on the distribution of p-11B reaction in a magnetic environment which considers both the effect of neutral beams and background plasma is in progress, as well as a detector simulation which provides the detection efficiency. Here we would like to discuss the feasibility of the varies types of detectors, which could tell energetic alpha particles from proton.

Key words: MCF; p-11B reaction; EHL-2; Spherical Torus; FILD; NPA; GRS; CR39