Feasibility study of a Coherence Imaging Charge Exchange Recombination Spectroscopy diagnostic for Wendelstein 7-X

R. López-Cansino1, V. Perseo2, E. Viezzer1, O.P. Ford2, D.M. Kriete3 and the W7-X team*

1 Department of Atomic, Molecular and Nuclear Physics, University of Seville, Seville, Spain

2 Max-Planck-Institut für Plasmaphysik, Greifswald, Germany

3 Auburn University, Auburn, Alabama, United States of America

* See author list of Thomas Sunn Pedersen et al 2022 Nucl. Fusion 62 042022

A feasibility study of a Coherence Imaging Charge Exchange Recombination Spectroscopy (CICERS) diagnostic has been carried out for the Wendelstein 7-X stellarator (W7-X). This diagnostic, based on Coherence Imaging Spectroscopy (CIS) [1] and Charge Exchange Recombination Spectroscopy [2], is expected to measure charge exchange radiation of the main impurity species present in the plasma. The spectral information of the radiation is encoded in a 2D fringe pattern, which is generated by birefringent plates, in order to obtain 2D maps of relevant impurity ion parameters such as rotation velocity, temperature and density after a demodulation procedure is applied to the fringe pattern.

A synthetic diagnostic has been developed, which enables the optimization of the design and components of the system, based on the already existing CIS diagnostic in W7-X [3], the latter optimized for impurity flow measurements from passive lines in the edge of the plasma. The CICERS system has been set up and characterized in the laboratory, finding good agreement in its behaviour when compared to the simulation results. This also increases the confidence of using the synthetic simulations in the calibration process of the diagnostic, which will be routinely calibrated with a laser before and after every measurement [4].

[1] J. Howard 2010 J. Phys. B: At. Mol. Opt. Phys. 43 144010

[2] R. J. Fonck 1985 Rev. Sci. Instrum. 56 855

[3] V. Perseo et al 2020 Rev. Sci. Instrum. 91 013501

[4] D. Gradic et al 2019 Fusion Eng. Des. 146 995–998