Novel multi-energy soft x-ray camera in the WEST tokamak: first data

and synthetic diagnostic

- Chellaï1, L. F. Delgado-Aparicio1, D. Vezinet 2,3, T. Barbui1, R. Dumont1, K. W. Hill1, P. Malard2, N. Pablant1, B. Stratton1, and the WEST team2

1Princeton Plasma Physics Laboratory, Princeton, NJ, USA

2IRFM CEA Cadarache, Saint Paul lez Durance, France

- Commonwealth Fusion Systems, Cambridge, MA, USA

During the C6 campaign, the tungsten (W) Environment in Steady-state Tokamak (WEST) tokamak was operated for the 1st time with a water-cooled full tungsten divertor -similar to that of ITER- and long-pulse scenarios, making it an ideal environment for high-Z impurity transport studies. In that context, a compact multi-energy (2-30 keV) soft x-ray diagnostic (ME-SXR) [1] was deployed by PPPL in WEST for high-Z impurity transport studies and electron temperature profile measurements. The ME-SXR consists of the PILATUS3 photon-counting detector manufactured by DECTRIS Ltd. mounted on a pinhole camera with a temporal and spatial resolution of 2 ms and 1-2 cm, respectively. The novelty of this soft x-ray diagnostic lies in the fact that the lower-energy threshold is set independently on each one of the 100k pixels with a high energy resolution (< 1 keV). The design, capabilities and engineering challenges of the ME-SXR diagnostic are briefly presented here.

This contribution mainly presents the first data of the ME-SXR diagnostic acquired during C6. A tentative comparison of the experimental x-ray emissivity with predictions made using the synthetic diagnostic based on the FLYCHK suite [2] for the computation of the charge-state distribution and x-ray emissivity of the plasmas as well as the ToFu [3] open-source python library will also be presented.

This work is supported by the U.S. DOE-OFES under Contract No. DE-AC02-09CH11466.

References

- O Chellai, LF Delgado-Aparicio, P VanMeter, T Barbui, J Wallace, KW Hill, N Pablant, B Stratton, C Disch, B Luethi, et al. Calibration of a versatile multi-energy soft x-ray diagnostic for west long pulse plasmas. Review of Scientific Instruments, 92(4):043509, 2021.

- H-K Chung, MH Chen, WL Morgan, Yu Ralchenko, and RW Lee. Flychk: Generalized population kinetics and spectral model for rapid spectroscopic analysis for all elements. High energy density physics, 1(1):3–12, 2005.

- D Vezinet. "https://github.com/tofuproject/tofu".